# Required Components

|  |  | BOM - Fubbles Light-Up Mini Bubble Machine - 3.5mm Mono Jack and Ring - 22 AWG Wire - 3 AA Batteries |
| --- | --- | --- |

# Required Tools

- Phillip screwdriver

- Ruler

- Wire strippers

- Soldering iron and solder

- Drill and ¼” drill bit

# Required Personal Protective Equipment (PPE)

- Safety glasses

# Assembly Instructions

| - Carefully remove the toy from its packaging without damaging it. Locate and remove the 4 screws from the bottom of the toy. Separate the toy into two halves. |  |
| --- | --- |
| - Remove the motor from the base. |  |
| - Cut both the red wires using wire cutters. |  |
| - Cut 2 pieces of wire, each approximately 4 cm long. |  |
| - Strip approximately 0.5cm off the end of each wire. |  |
| - Solder the two wires onto the front two tabs on the mono jack. |  |
| - Tin the other two ends of the wires by adding a small amount of solder to each wire. |  |
| - To ‘close the loop,’ solder a wire to an existing red wire and solder the second wire to the second red wire. |  |
| - Using a ¼” drill bit, drill a hole into the side of the toy. Remove any loose plastic. Secure the mono jack to the case by screwing on the mono jack ring from the outside. |  |
| - Test the toy by installing batteries and plugging an assistive switch into the mono jack. Note: If the toy does not turn on, please check the soldered connections. |  |
| - Reassemble the toy by placing the motor back and reattaching the toy shell. Re-test the toy with the assistive switch. If it still works properly, re-install the toy into its original packaging. |  |