# Required Components

|  |  | BOM - Gazillion Hurricane Bubble Machine - 3.5 mm Mono Jack and Ring - 22 AWG Wire - 6 AA Batteries |
| --- | --- | --- |

# Required Tools

- Triangle screwdriver

- Ruler

- Wire strippers

- Soldering iron and solder

- Drill and 1/4” drill bit

# Required Personal Protective Equipment (PPE)

- Safety glasses

| Step 1: Partially Unbox the Gazillion Open the top of the box, but keep the toy on the cardboard base. This will make it easier to reassemble after modification |  |
| --- | --- |
| Step 2: Cut the Battery Cover Nylon Strap Using the flush cutters cut the nylon strap, and discard it. Set the Battery cover aside. |  |
| Step 3: Open the Battery Case Make sure to keep the screws together. |  |
| Step 4: Remove The Battery Tray Using the 2 mm Triangle Screwdriver (or 2.4 Flathead screwdriver) unscrew the 4 screws holding the battery tray in place. Be careful not to break the wires attached to the back of the battery tray. |  |
| Step 5: Remove the Switch Lift the switch out of the toy, and using the flush cutters, cut the wires |  |
| Step 6: Strip the Wires Using the wire strippers, strip the insulation from the switch leads so about 1 cm of the metal wire is exposed. |  |
| Step 7: Solder the Wires to the Switch Jack Solder the wires to the 2 legs closest to the threaded end. |  |
| Step 8: Test the Toy Insert batteries into the Battery Tray and insert a switch into the Mono Jack Press the Switch: You should hear the motor turn on Remove the batteries and the switch |  |
| Step 9: Insert Jack into Switch Hole Insert the mono jack into the hole where the switch was originally located. Use flush cutters to clear any plastic that might block the jack. |  |
| Step 10: Install the Jack Nut Using the edge of a flathead screwdriver turn the nut until tight. Use the slots cut into the Jack Nut to tighten. |  |
| Step 11: Install the Battery Tray Using the 2 mm Triangle Screwdriver (or 2.4 Flathead screwdriver), install the battery tray screws |  |
| Step 12: Install the Batteries and Battery Cover |  |
| Step 13: Replace & Tape the Top Cover Place a strip of electrical across the Top Cover to keep in place during shipping. This also helps us know that the toy is ready for delivery. |  |
| Step 14: Replace it in the Box Carefully re-box the toy into its original packaging, and secure the top flap with Clear Tape. |  |