# Required Components

|  |  | BOM - WeOfferWhatYouWant First Guitar Toy - Mono Jack (x4) - 22 AWG Wire - 10 nF Capacitors (x2) |
| --- | --- | --- |

# Required Tools

- Small Phillips screwdriver

- Drill and 1/4” drill bit

- Wire strippers

- Soldering iron and solder

- File (optional)

# Required Personal Protective Equipment (PPE)

- Safety glasses

# Assembly Instructions

| - Turn over the toy and remove all 9 screws. |  |
| --- | --- |
| - Remove the silicone button. You should be able to pull the two silicone legs out of the small holes. |  |
| - Cut 4 lengths of wire about 10cm long |  |
| - Strip approximately 0.5cm off of the small wires. | 0.5 cm |
| - Solder a wire to the first leg of a mono jack, solder a second wire to the second leg of the mono jack. Repeat with the second mono jack and other wires |  |
| - Prior to soldering to the device, tin your wire. The best way to do this is cover the exposed wire end in solder. |  |
| - Solder the two wires of the jack onto the circuit board as shown. Any wire of the two wires will work. Repeat on all four sets of points. Note: It is easy to melt the board; use caution. |  |
| - Solder the capacitors onto the two jacks in the centre of the guitar. |  |
| - Drill 1 hole on the top of the casing as directed in the photos. |  |
| - Drill 3 remaining holes in the bottom casing. Ensure you are spacing out space for the jacks. |  |
| - Plug in an assistive switch and test all 4 jacks. If the toy doesn’t turn on when the switch is activated, check the soldered connections. |  |
| - If the toy is working properly, insert the mono jacks through the drilled holes and screw the rings on the outside of the toy. |  |
| - Place the toy back together and re-insert the screws that were removed earlier, being careful not to pinch any wires. Re-test the toy to ensure it is still functioning properly. |  |