# Required Components

|  |  | BOM - Amazon LED Electric Duck Climbing Stairs Toy - Mono jacks x2 - 22 AWG Wire - 2 AA Batteries |
| --- | --- | --- |

# Required Tools

- Screwdriver

- Soldering iron

- Wire stripper

- Flush cutter

- Drill and ¼ drill bit

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

## Step 1

|  | - Carefully remove toy from packaging, the toy will be put back in packaging (if possible) after it is adapted - This toy has a lot of parts – ducks and a track that can be kept in the box - Using a screwdriver, turns screws indicated in the red circles to open the toy – including the screw inside the batter compartment - Set screws aside in a safe place |
| --- | --- |

## Step 2

|  | - After screws are undone, gently pull the toy into two halves. Be very careful while doing this as to not pull off any existing wires or connections - If some parts move, just take note of how everything fits together but they can also be set aside while you work on the toy. |
| --- | --- |

## Step 3

|  | Next we will prepare the mono jack and wires. Cut 2 pieces of wire each 8 cms in length Strip both ends of both wires, leaving about .5 cm of exposed wire on each end |  |
| --- | --- | --- |

## Step 4

|  | - Pick one wire, slide exposed wire into first metal arm on mono jack (the arm closest to the circular jack) - With second wire, slide one end of exposed wire into the middle metal arm on mono jack - Solder wire to mono jack Please Note: Check that you have the correct metal arms of mono jack, refer to picture |
| --- | --- |

## Step 5

|  | - Tin the ends of your wire. The best way to do this is cover the exposed wire end in solder. - This will help you stick the wires to an existing spot where the is solder |
| --- | --- |

## Step 6

|  | - Locate where the on/off switch is located. Undo the screws and gently pull the switch to give yourself space to work with it - For this assembly, we are attaching our mono jack + wires to the same spots on the original switch |
| --- | --- |

## Step 7

|  | - Using the wires and mono jack you previously prepared, you will solder the ends of your new wire to where the current red wires are. - You will have one new wire and one old wire coming from each spot indicated in the circles - Test your connections using an assistive switch. Listen for the motor to start when the switch is activated. Please note: you will need batteries to test |
| --- | --- |

## Step 8

|  | - Using a ¼ inch drill bit, drill a hole to the side of the battery compartment. - Check that the mono jack end fits through the hole and that it does not interfere with any mechanism of the toy. - Add the ring to the outside of the toy to hold the mono jack in place. |
| --- | --- |

## Step 9

|  | - Reassemble the original switch to its original position, close the toy – checking that no wires are being pinched and the mechanics are moving. - Reassemble by adding the screws to their original spots. Test the toy again once reassembled. |
| --- | --- |