# Required Components

| 1 3 2 | BILL OF MATERIALS 1 Boppin’ Beaver Linkimal 2 Mono jack 3 Wire |
| --- | --- |

# Required Tools

- Drill with ¼” drill bit

- Wire Strippers

- Flush Cutters

- Phillips screwdriver

- Soldering Iron

- Solder

- Ruler

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

## Step 1

x 2

Cut 2 lengths of wire about 11cm long. Strip 0.5cm of the insulation off each end of the wire.

## Step 2

Solder a wire to the first leg of a mono jack, solder a second wire to the second leg of the mono jack.

## Step 3

Carefully take the Boppin’ Beaver out of the packaging without damaging it. The toy will be put back in the packaging once adapted.

## Step 4

Remove the 4 screws from the body of the beaver including the one hidden in this picture by the beaver’s tail.

## Step 5

Disconnect the white speaker wire that is holding the 2 halves of the beaver together. Set the half of the beaver with the head attached aside.

## Step 6

Drill a ¼” hole in the side of the beaver.

## Step 7

Remove the 2 screws holding down the circuit board. Remove the turquoise switch and set it aside.

## Step 8

Solder the other ends of the wires attached the mono jacks to the locations shown below.

## Step 9

Remove the nut from the mono jack and push the mono jack through the hole from the inside. Reinstall the nut and tighten to secure the mono jack.

## Step 10

Reinstall the turquoise switch and screw in the 2 screws into the circuit board.

## Step 11

Reattach the speaker wire and screw in the 4 screws into the back of the beaver.

## Step 12

Put the Boppin’ Beaver back into the original packaging.