# Required Components

|  |  | BOM - Dan&Darci Paint Spin Art Machine - 1 Mono jack and nut - 22 AWG Wire - 4 AA Batteries Refer to the Tool and Component List for purchasing details. |
| --- | --- | --- |

# Required Tools

- Phillips Screwdriver

- Wire stripper

- Flush cutter

- Soldering iron and stand

- Drill and ¼ inch bit

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

## Step 1

|  | - Carefully cut the tape and open the box - Set the paints and markers aside, we will reassemble the kit once the toy is adapted |
| --- | --- |

## Step 2

|  | - On the bottom of the toy, locate the black foam pads, lift these up and set aside - Utilizing a screwdriver, turns screws indicated in the red arrows to remove bottom covering - Set screws aside in a safe place |
| --- | --- |

## Step 3

|  | - After the screws are out, you can gently pull the toy open. It will not come completely apart, but enough to access the inside of the toy. - Locate the original on/off switch and pull it out of its original spot |
| --- | --- |

## Step 4

|  | - Cut two pieces of wire about 15 cm long. Using wire strippers, remove the plastic insulation from the last 1 cm on both ends of each wire |
| --- | --- |

## Step 5

|  | - Pick one wire, slide exposed wire into first metal arm on mono jack (the arm closest to the circular jack) - With second wire, slide one end of exposed wire into the middle metal arm on mono jack - Solder wire to mono jack - Please Note: Check that you have the correct metal arms of mono jack, refer to picture |
| --- | --- |

## Step 6

|  | - Tin the ends of your wire. The best way to do this is cover the exposed wire end in solder - This will help you stick the wires to an existing spot where there is solder |
| --- | --- |

## Step 7

|  | - You will now connect the wires to the original switch - While holding one wire end (which you just tinned) onto the metal tab, use your iron to heat up the original solder and the new solder, merging the solder - Let the solder dry, leaving the original wire and new wire attached to the original switch Important: Add batteries and test that your toy works with your assistive switch |
| --- | --- |

## Step 8

|  | - Drill a hole in the plastic enclosure with a ¼” drill bit, in the location noted in the picture |
| --- | --- |

## Step 9

|  | - Push the jack through the enclosure and install the Jack Nut on the outside - This will keep the jack in place during use |
| --- | --- |

## Step 10

|  | - Place the yellow switch cap, then using the Phillips head screwdriver, reinstall the six screws |
| --- | --- |

## Step 11

|  | - Carefully re-box the toy into its original packaging and secure the top flap with Clear Tape. |
| --- | --- |