# Required Components

|  |  | BOM - Playskool Stack ‘n Spin Monkey Gears Toy - Female/Male Mono Cable - Cable Tie - AA Batteries (x3) Refer to the Tool and Component List for purchasing details. |
| --- | --- | --- |

# Required Tools

- Flat head screwdriver

- Possibly a Triangle Screwdriver

- Wire strippers

- Soldering iron and solder

- No Clean Flux Paste

- Drill and 1/8” drill bit

- Glue Gun

# Required Personal Protective Equipment (PPE)

- Safety glasses

# Assembly Instructions

| - Flip the toy over and remove the screws on the back panel. Place screws in a safe space for reassembly. Note: Some of the screws may be a TA Triangle Screw that can be removed with a Triangle Head Screwdriver |  |
| --- | --- |
| - Carefully lift and remove the back panel of the toy. |  |
| - With a 1/8 drill bit, drill a hole on the side of the back panel. Note: Ensure you are drilling in a spot that won’t impact reassembly. |  |
| - Thread the female cable through the pre-drilled hole. Note: some cables come pre-wire stripped. If not please wire strip your cables. |  |
| - On the base of the toy find the circuit board and remove the two remaining screws and the pink power switch. Put the screws and switch in a safe space for reassembly. |  |
| - Turn the circuit board over. In the image you will see the button we will solder to by the circle on the image. There will be a silicone cover on it. Please remove. - Dab flux on the two contact points. Add a small globe of solder to the two spots. |  |
| - Tin each exposed end of wire with solder. |  |
| - Solder the two wires to either side of the button contacts. Note: colour does not matter to each spot. |  |
| - To help hold the connection add glue to the soldered points. |  |
| - Flip the circuit board right side up. Reinstall the two screws and the power switch. |  |
| - Add a small zip tie to the female cable to prevent it from being pulled out. |  |
| - Reassemble the screws. |  |
| - Test Toy! |  |